Upande Irrigation

How a week of irrigation is planned, scheduled and executed

End-to-end process reference  ·  3 August 2026  ·  upande_irrigation app, Frappe v16

This document describes the complete irrigation planning process as it now works: what is measured, how demand is calculated for every section, how a shared pump is shared out, and how the result reaches an operator. It covers the doctypes involved, the arithmetic at each step with a fully worked example from live data, the rules under which the system refuses to plan, and what changed in the August 2026 rebuild.

# 1. The process in one page

Irrigation is planned one week at a time, one shift at a time. The week runs Thursday to Wednesday. The cycle has five stages, and each stage has exactly one owner.

| Stage | What happens | Owner | Runs |
| --- | --- | --- | --- |
| 1. Measure | A daily Weather Reading records pan evaporation, rainfall and temperature per farm. Pan cups become millimetres; ET pan becomes ET crop. | events/weather_reading.py | Daily, on save |
| 2. Qualify | Before anything is created, the farm's elapsed week is checked for completeness. An incomplete week is refused and the reason recorded. | engine/demand.py | Weekly, per farm |
| 3. Demand | For each shift: apply the crop coefficient, subtract rainfall, add whatever the shift still owes, convert millimetres to hours. | engine/demand.py | Weekly, per shift |
| 4. Allocate | Group shifts by the pump that serves them. Scale every request to fit the pump's weekly capacity. Split into cycles, place them in the week. | engine/allocate.py | Weekly, per pump |
| 5. Execute | The operator sees which shift is running now, what is queued, and what is carrying over. Unmet water becomes next week's opening debt. | api/scheduler.py, dashboard | Continuous |

The one rule that governs everything Demand is measured over the week that has already happened — never the week being planned. A plan generated on Friday for the week starting the following Thursday measures the seven days ending the day before that Thursday. Those days are settled; the planned week has not happened yet and contains no data.

# 2. The irrigation hierarchy

Every calculation is anchored to a position in this hierarchy. The unit that is planned and scheduled is the shift, because a shift is what a pump can actually run at one time.

Farm Lokitela weather is recorded here └─ Section 23HA_SECTION a pump serves a section └─ Shift 23HA - SHIFT 1 THE PLANNED UNIT └─ Block AIRSTRIP BLK 4 - KL the valves that open

A section's shifts are numbered. Lokitela currently has four sections and 55 active shifts:

| Section | Active shifts |
| --- | --- |
| 23HA | 7 |
| 56HA | 15 |
| 65HA | 14 |
| 70HA | 19 |
| Total | 55 |

A shift waters its own trees. It therefore needs the full weekly deficit — not a share of it. What the shifts share is the pump, and that is a constraint on delivery, not on need. Keeping those two ideas apart is the central design decision in the rebuild: demand is what the trees lost, allocation is what the infrastructure can give back.

# 3. Doctypes involved

| Doctype | Role in the process | Written by |
| --- | --- | --- |
| Farm | The estate. Carries is_irrigation_farm, which decides whether the scheduler visits it. | Manual |
| Warehouse (type Section) | A section. Named <PREFIX>_SECTION. Links a pump profile to a group of shifts. | Manual |
| Block Type | A shift, named <SECTION> - SHIFT <N>. is_active decides whether it is planned. | Manual |
| Blocks List (child) | The blocks a shift waters — the valves that open. | Manual |
| Weather Reading | One row per farm per day: pan cups, rainfall, min/max temperature. Derives ET pan and ET crop on save. | Daily entry |
| Irrigation Settings (single) | Every coefficient and default: crop coefficient, application rate, coverage, cycle thresholds, minimum weather days. | Manual |
| Irrigation Pump Profile | Weekly water target and pump flow rate per section. Capacity in hours is target ÷ flow. | Manual |
| Irrigation Scheduler (single) | When the week starts, which week to plan, which farms, error tolerance. | Manual |
| Irrigation Planner | One document per shift per week. Holds the measured window, the demand, the allocation and the outcome. | Engine |
| Irrigation Scheduler Run | The audit record of one run: the full log, per-shift results, and every farm-week that was refused. | Engine |

Removed in this rebuild The Irrigation Calculation child table (field irrigation_calculations) is gone. It presented an editable per-block application rate and coverage, but those values never reached the calculation — hours always came from the farm defaults. Its client script also multiplied by coverage where the server divides, so a row's hours never matched the header's. Editable fields that change nothing are worse than no fields at all.

# 4. Stage 1 — Measuring the week

A Weather Reading is entered once a day per farm. Two derived figures are computed on save, and every downstream number depends on them.

pan_depth_mm = pan_cups x depth_per_cup_mm depth_per_cup_mm = 0.5 et_pan = rainfall_mm + pan_depth_mm et_crop = et_pan x pan_to_crop_coefficient pan_to_crop = 0.95

ET pan adds rainfall back because the pan loses water to evaporation and gains it from rain; the gross atmospheric demand is the sum. The pan-to-crop coefficient converts an open water surface into a crop canopy.

A temperature of 0 °C is treated as not recorded, not as a real measurement. Averaging zeros in would drag the mean down and suppress the anthracnose index.

# 5. Stage 2 — Qualifying the week

The seven days of the measured week are counted before any planner is created. This is one check per farm, because every shift on a farm shares the same weather.

measured_to = plan_from - 1 day measured_from = measured_to - 6 days if days_with_readings < min_weather_days: REFUSE the whole farm-week

The threshold is min_weather_days in Irrigation Settings, default 7 — so a single missed day refuses the week. A refused farm-week produces no planners at all. The refusal and its reason are written to the Irrigation Scheduler Run.

Why refusing matters more than it sounds Under the old engine an unmeasurable week still produced a planner for every shift, and every one of them read "No irrigation required" — indistinguishable from a genuinely wet week. Silence and zero are different answers, and the system must not confuse them.

The live site demonstrates both outcomes. Weather for Lokitela ends 2026-07-12:

| Plan week | Measured week | Readings found | Outcome |
| --- | --- | --- | --- |
| 2026-07-02 | 2026-06-25 → 2026-07-01 | 7 of 7 | Planned — 55 planners |
| 2026-07-09 | 2026-07-02 → 2026-07-08 | 6 of 7 (2026-07-03 missing) | Refused |
| 2026-07-16 | 2026-07-09 → 2026-07-15 | 4 of 7 | Refused |

# 6. Stage 3 — Demand, for every section

Demand is computed identically for every shift in every section. The inputs are the measured week's totals, the crop coefficient, and whatever the shift still owes from before.

## 6.1 The arithmetic

clean_et_crop = SUM(et_crop over measured week) x et_crop_coefficient week_deficit = max(0, clean_et_crop - SUM(rainfall over measured week)) total_needed = week_deficit + carried_deficit required_hours = total_needed / application_rate_mm_hr / (coverage / 100)

| Term | Meaning | Current value |
| --- | --- | --- |
| et_crop_coefficient | Crop utilisation — the fraction of atmospheric demand the canopy actually transpires. | 0.65 |
| application_rate_mm_hr | Depth the emitters apply per hour. | 2.8 mm/hr |
| coverage | Fraction of the block the emitters wet. Dividing by it lengthens the run, because only part of the ground receives water. | 70 % |
| carried_deficit | Unmet millimetres from the most recent prior planner for this shift. | per shift |

## 6.2 Carry-forward

A shift that was not given enough hours ends the week in debt. That debt opens the next week's demand. The lookup takes the most recent prior planner for the shift, ordered by date — not the planner dated exactly seven days earlier. A skipped week no longer writes the debt off.

On the regenerated 27 weeks the debt behaves as it should: it builds through the dry start of the year to a peak of 97.3 mm in mid-February, then drains as wetter weeks arrive and the granted hours chip away at it. It does not run away.

## 6.3 Disease risk, on the same window

The anthracnose index is computed from the same measured week, so it can never disagree with the irrigation figures about which days it is describing.

Z = -58.99 + (3.22 x mean_temp) + (0.18 x weekly_rainfall) Z >= 20 High Risk - Fungicide Required Z >= 15 Infection Risk - Monitor Closely Z >= 5 Spore Release - Low Alert else Low Risk

# 7. Stage 4 — Allocating the pump

Demand is what the trees lost. Allocation is what the pump can return. A single shift cannot compute this, because the answer depends on every other shift drawing on the same pump — which is why allocation is the scheduler's job and not the planner's.

## 7.1 Capacity

capacity_hours = water_target_m3_per_week / pump_flow_rate_m3_per_hr no profile, or either figure missing: 168 hr/wk, flagged UNVERIFIED

With no Irrigation Pump Profile there is no capacity to allocate against. The engine assumes a full week and says so on every affected planner. It never presents an assumed number as a limit. The live site currently has zero pump profiles, so this is the active path.

## 7.2 Grouping

Shifts are grouped by the pump named in their section's profile. Sections sharing a pump are sequenced against one capacity and one timeline. Sections with no profile share a single unknown-pump group — the pessimistic reading, because an unknown pump might be one pump, and scheduling them concurrently could overdraw a main that cannot carry it.

## 7.3 Proportional capping

total = SUM(required_hours across the pump's shifts) factor = capacity_hours / total (only when total > capacity) granted = required_hours x factor

Every shift is scaled by the same factor, so shares stay proportional to need and a shift needing nothing reserves nothing. The old rule divided capacity by the number of shifts, which handed an idle shift the same slice as a parched one and then discarded the unused slice.

## 7.4 Cycles and placement

count = ceil(granted / auto_cycle_threshold_hrs) floored at cycles_when_above or cycles_when_below when granted <= threshold each = granted / count cycles are separated by cycle_rest_hours default 2.0 hr shifts on one pump are placed end to end, never overlapping a cycle that will not fit inside the week is not placed - it carries

Splitting a long run lets water infiltrate instead of running off. The old engine placed cycles back to back, which is not cycling, and anchored every section at hour 0, so sections ran concurrently while capacity was being capped per section.

# 8. Worked example — 23HA · SHIFT 1

Live record IRPL-2026-01019, plan week 2026-01-08 → 2026-01-14, farm Lokitela. Every figure below is read from the database, not constructed.

| Step | Calculation | Result |
| --- | --- | --- |
| Measured window | plan_from 2026-01-08, minus one day, back seven days | 2026-01-01 → 2026-01-07 |
| Completeness | readings found vs min_weather_days | 7 of 7 — proceed |
| Weather totals | summed over the measured week | rain 0.00 mm, ET pan 34.50 mm, ET crop 32.775 mm |
| Clean ET crop | 32.775 × 0.65 | 21.3038 mm |
| Week deficit | max(0, 21.3038 − 0.00) | 21.3038 mm |
| Carried debt | most recent prior planner for this shift | 0.00 mm — first week |
| Total needed | 21.3038 + 0.00 | 21.3038 mm |
| Required hours | 21.3038 ÷ 2.8 ÷ 0.70 | 10.87 hr |
| Pump capacity | no profile for this section | 168 hr/wk, unverified |
| Capping factor | 168 ÷ (55 shifts × 10.87 hr) = 168 ÷ 597.8 | 0.281 |
| Granted hours | 10.87 × 0.281 | 3.05 hr |
| Cycles | 3.05 ≤ 5.0 hr threshold → cycles_when_below | 1 × 3.05 hr |
| Window | first in the unknown-pump queue | 2026-01-08 00:00 → 03:03 |
| Delivered depth | 3.05 × 2.8 × 0.70 | 5.99 mm |
| Unmet | 21.3038 − 5.99 | 15.32 mm — carries to 2026-01-15 |
| Disease index | Z from mean temp and 0.00 mm rain | 5.18 — Spore Release, Low Alert |

What this record is telling the farm 23HA · SHIFT 1 lost 21.3 mm and received 6.0 mm. It is not a scheduling error — it is a capacity statement. With 55 shifts sharing one assumed 168 hr week, the farm can return about 28 % of what it loses in a dry week. Creating the Irrigation Pump Profiles is what turns that assumption into a measured fact, and may well raise the ceiling.

# 9. What each field means and who writes it

Demand fields and allocation fields are written by different stages. Opening a scheduled planner and pressing Save recomputes the demand block only — it no longer overwrites the scheduler's allocation, which was a defect in the old engine.

| Field | Meaning | Written by |
| --- | --- | --- |
| measured_from / measured_to | The elapsed week the demand came from. | Planner hook |
| weather_completeness | "7 of 7 days logged". | Planner hook |
| weekly_rainfall_mm, weekly_et_pan_mm, raw_et_crop_mm | Measured-week totals. | Planner hook |
| clean_et_crop_mm | ET crop after the crop coefficient. | Planner hook |
| this_week_deficit | Clean ET crop minus rainfall, floored at zero. | Planner hook |
| carried_deficit_mm | Debt opened from the prior planner. | Planner hook |
| total_water_needed_mm | This week plus carried. | Planner hook |
| required_hours | Hours to meet demand, before any cap. | Planner hook |
| z_value, z_risk_level | Anthracnose index and band. | Planner hook |
| shift_hours | Hours actually granted by the allocator. | Scheduler |
| cycles_count, cycle_hours_each, cycle_plan | How the run is split. | Scheduler |
| scheduled_start / scheduled_end | First and last moment of the run. | Scheduler |
| delivered_depth_mm | Millimetres the granted hours deliver. | Scheduler |
| unmet_deficit_mm | Shortfall — becomes next week's carried debt. | Scheduler |
| capacity_warning | Why the grant fell short, in plain words. | Scheduler |

# 10. Operating the system

## 10.1 Weekly cadence

Enter a Weather Reading every day. This is the single input the whole model rests on; a missing day refuses that farm's next plan.

The scheduler runs automatically on Friday at 06:00 (cron 0 6 * * 5), planning the week set in Irrigation Scheduler.

Review the Irrigation Scheduler Run: it lists every planner created, every farm-week refused and why, and the capacity warning on each shift.

Operators work from the dashboard at /upande-irrigation — what is running now, what is queued, and what is carrying over.

## 10.2 Running it by hand

# plan the configured week now bench --site <site> execute upande_irrigation.api.scheduler.run \ --kwargs "{'triggered_by':'Manual'}" # plan one specific week bench --site <site> execute upande_irrigation.api.scheduler.run \ --kwargs "{'for_week':'2026-01-08'}" # regenerate a range of historical weeks, one run each bench --site <site> execute upande_irrigation.api.scheduler.backfill \ --kwargs "{'from_week':'2026-01-01','to_week':'2026-07-16'}"

## 10.3 Settings that change the outcome

| Setting | Effect | Current |
| --- | --- | --- |
| min_weather_days | How complete a week must be before it can be planned. | 7 |
| default_avocado_utilisation | Crop coefficient applied to ET crop. | 0.65 |
| default_application_rate_mm_hr | Emitter output — divides the deficit into hours. | 2.8 |
| default_irrigation_coverage | Wetted fraction — lengthens the run. | 70 % |
| auto_cycle_threshold_hrs | Longest single cycle before a run is split. | 5.0 |
| cycle_rest_hours | Gap between a shift's cycles. | 2.0 |
| organisation_name | Estate name shown in the dashboard header. | Kaitet Group |

# 11. What changed, and the evidence

The review examined 836 planners across 29 weeks. 819 of them — 98 % — instructed the farm not to irrigate. The arithmetic was never wrong; it was aimed at the wrong week.

| Defect | Evidence | Fix |
| --- | --- | --- |
| Demand measured the week being planned, which had not happened yet. DATEDIFF(from_date, creation) = 6 on every batch. | 819 of 836 planners had zero hours; 709 were computed against "0 of 7 days logged". | Demand now measures the seven days ending the day before the plan week. |
| An unmeasurable week still produced planners reading "No irrigation required". | Indistinguishable from a genuinely wet week. | The farm-week is refused and the reason recorded. |
| The Before Save hook computed both demand and allocation, so saving a scheduled planner overwrote the grant. | shift_hours recomputed uncapped on every save. | Hook writes demand only; scheduler writes allocation. |
| The migrated Server Scripts were never retired, so two engines ran. | "Irrigation Planner — Compute Shift" was enabled alongside the Python hook, and the old cron fired at the same 0 6 * * 5. | All five superseded Server Scripts disabled by patch. |
| Capacity divided equally across shifts. | An idle shift reserved as much as a parched one; the unused slice was discarded. | Proportional scaling by capacity ÷ total demand. |
| Every section anchored at hour 0 while capping was per-section. | Sections sharing a pump each assumed the whole week. | Shifts grouped by pump and sequenced in one queue. |
| Cycles ran back to back. | No infiltration gap — not cycling at all. | Cycles separated by cycle_rest_hours. |
| Carry-forward required to_date = from_date − 1 exactly. | One skipped week wrote the debt off silently. | Most-recent-prior-planner lookup, ordered by date. |
| Per-block overrides never affected the result. | Row mm_hr never reached the calculation; the client script multiplied by coverage where the server divides. | Child table and its doctype removed. |

## 11.1 Before and after, on the same site

| Measure | Before | After |
| --- | --- | --- |
| Planners | 836 across 29 weeks | 1485 across 27 weeks |
| With real demand | 17 (2 %) | 1430 (96 %) |
| Computed against no weather | 709 | 0 — those weeks are refused |
| Weeks refused for incomplete data | 0 | 2 (2026-07-09, 2026-07-16) |
| Peak carried debt | not tracked meaningfully | 97.3 mm mid-February, then drains |
| Automated tests | none | 52, passing |

# 12. Known limits and next steps

No Irrigation Pump Profile exists on the live site. Until one is created per section, capacity is an assumed 168 hr/wk, every planner carries an "unverified" warning, and all sections are sequenced as though they share one pump. This is the single highest-value data entry task outstanding.

Every shift on a farm currently produces identical demand, because nothing in the model is shift-specific — no per-shift area, crop age or coefficient. Proportional capping therefore degenerates to equal shares. Adding per-shift area would make the allocation genuinely differentiated.

Weather data ends 2026-07-12, so the current week cannot be planned. The dashboard reports this as an alert rather than showing zeros.

Phase 2 of the rebuild covers visualisation: the week Gantt, pump load, water balance and a calculation explainer that shows an operator exactly how a shift's hours were derived.

Carry-forward is uncapped by design. It drains correctly on the current data, but a physical ceiling tied to root-zone holding capacity would be more defensible if a farm ever ran a long structural shortfall.

# Appendix — Source map

| File | Responsibility |
| --- | --- |
| upande_irrigation/engine/demand.py | Pure. Measured window, aggregation, plannability, deficit, required hours, anthracnose index. |
| upande_irrigation/engine/allocate.py | Pure. Pump capacity, proportional granting, cycle planning, window placement. |
| upande_irrigation/events/irrigation_planner.py | Before Save adapter. Fetches readings, writes the demand block, carry-forward and persistence streak. |
| upande_irrigation/api/scheduler.py | Run and backfill. Refusal, pump grouping, allocation, warnings, audit record, live_sections. |
| upande_irrigation/api/overview.py | Dashboard read model: tiles, alerts, schedule grid, deficit trend. |
| upande_irrigation/tests/ | 52 tests. Each module's docstring names the defect it locks out. |
| upande_irrigation/patches/v1_0/add_engine_fields.py | Adds required_hours and the measured window; drops the per-block table. |
| upande_irrigation/patches/v1_0/retire_superseded_server_scripts.py | Disables the five Server Scripts the Python engine replaced. |

Run the tests with:

bench --site <site> run-tests --app upande_irrigation